DIGITAL DESIGN PROJECT

Secure UART Communication System

UART Interface with Hardware AES-128 Encryption and CRC-16/XMODEM Error Detection

Prepared by: Mina Moawad , Muhammed Afifi , Hoda Ayman

Program:  Digital Design Phase

Supervisor:  Eng/Muhammed Salah

Institution: NTI

Date: 7/9/2026

Contents

	Document Control	4

	Abstract:	4

Introduction	5

1.	Project overview:	5

1.1	project objectives:	5

1.2	Functional Summary:	5

System Requirements:	6

2.1 Functional Requirements:	6

2.2 Assumed/ Configurable Parameters:	6

Literature Review	7

3.1 AES Integration with UART:	7

3.2 Error Detection over Serial Links:	7

3.3 Positioning of This Project:	8

System Architecture	8

4.1 Transmit Path:	8

4.2 Block-Level Responsibilities:	9

4.3 Positioning of This Project:	10

4.4 Byte Serialization of AES Result:	10

UART Communication Module	11

5.1 UART Transmitter Operation:	11

5.2 UART Receiver Operation:	11

5.3 UART Interface:	11

5.4 UART Testbench:	13

AES-128 Encryption Module	14

6.1 AES-128 Encryption Structure:	14

6.2 AES State Transformation:	15

6.3 AES Key Expansion:	16

6.4 AES Control Sequence:	16

6.5 AES Interface	17

6.6 AES Testbench:	17

CRC-16/XMODEM Module	19

7.1 CRC-16/XMODEM Parameters:	19

7.2 CRC Calculation Concept:	19

7.3 CRC Interface:	20

7.4 CRC Processing Architecture:	20

7.5 CRC Testbench:	21

End-to-End Data Path and Integration	22

8.1 Recommended Transmission Sequence:	22

8.2 Integrated State Machine:	22

Control and Handshake Strategy	23

9.1 Busy Protection:	23

9.2 Done Pulse Handling:	23

AES Integration with CRC	24

Timing and Synthesizability Considerations	26

10.1 AES Timing:	26

10.2 CRC Timing:	26

10.3 UART Timing:	26

10.4 Reset Strategy:	26

Design Decisions and Engineering Rationale	27

11.1 Security and Integrity:	27

11.2 CRC After Encryption:	27

Limitations and Future Improvements	28

Final Specifications	28

Conclusion	29

References	29

- Document Control

| Document Item | Value |
| --- | --- |
| Project Scope | UART communication with hardware AES-128 encryption and CRC-16/XMODEM integrity checking |
| Implementation | Verilog HDL / RTL |
| Cryptographic Algorithm | AES-128 encryption |
| Integrity Algorithm | CRC-16/XMODEM |
| Communication | UART |
| System Status | Individual modules implemented and integrated into a complete RTL system |
| Document Type | Technical Design, Architecture, Operation and Verification Report |

- Abstract:

This report documents a complete hardware communication and security data path implemented in Verilog/SystemVerilog HDL. The design combines a UART communication interface, an AES-128 encryption engine, and a CRC-16/XMODEM error-detection engine into an integrated RTL system. Application data is encrypted with AES-128, a CRC-16/XMODEM checksum is calculated over the resulting ciphertext, and the protected frame is transmitted serially through UART. The report covers the project's motivation and requirements, a review of related work, the system architecture, the detailed design of each RTL module (UART, AES-128, CRC-16/XMODEM), the end-to-end integration and handshake strategy, the verification plan (including the AES-128 known-answer vector and the CRC-16/XMODEM standard check value).

# Introduction

- Project overview:

This project implements a complete hardware communication and security datapath in Verilog/SystemVerilog HDL. The design combines a UART communication interface, an AES-128 encryption engine, and a CRC-16/XMODEM error-detection engine into an integrated RTL system. The objective is to demonstrate how communication, confidentiality, and data-integrity functions can be implemented and coordinated at the hardware level.

The system accepts application data, encrypts the data using AES-128, calculates a CRC-16/XMODEM checksum over the defined protected data field, and transmits the resulting frame through UART. On the receiving side, the corresponding data can be recovered and checked using the matching protocol. The integrated architecture therefore provides three distinct functions: reliable serial transport, confidentiality, and error detection.

  - project objectives:

- Implement a synthesizable UART transmitter/receiver subsystem in Verilog/SystemVerilog HDL.

- Implement AES-128 encryption as a hardware RTL block.

- Implement CRC-16/XMODEM using the standard CRC parameters and verify its calculation.

- Integrate the three functions into a single end-to-end communication datapath.

- Define clear start/busy/done handshaking between processing stages.

- Verify individual modules and the integrated system using simulation-based testbenches.

- Produce an RTL architecture that can be synthesized and used as a foundation for FPGA or ASIC implementation.

  - Functional Summary:

| Block | Primary Function | Main Data | Completion / Status |
| --- | --- | --- | --- |
| UART | Serial communication | Parallel byte ↔ serial bitstream | busy / done and RX indication |
| AES-128 | Confidentiality | 128-bit plaintext + 128-bit key → 128-bit ciphertext | busy / done |
| CRC-16/XMODEM | Integrity / error detection | Input data → 16-bit CRC | busy / done |
| Top-Level Integration | Sequencing and framing | End-to-end protected payload | System control / status |

# System Requirements:

## 2.1 Functional Requirements:

- The UART shall serialize and deserialize data according to the selected baud-rate configuration.

- The UART transmitter shall append the required start and stop bits to each transmitted byte.

- The UART receiver shall detect the incoming frame and reconstruct the received byte.

- The AES block shall perform AES-128 encryption on a 128-bit plaintext using a 128-bit master key.

- The CRC block shall calculate a 16-bit CRC using the CRC-16/XMODEM algorithm.

- The integrated controller shall prevent downstream stages from consuming incomplete or invalid data.

- The integrated system shall indicate completion and/or availability of results through defined status signals.

## 2.2 Assumed/ Configurable Parameters:

| Parameter | Description | Project Treatment |
| --- | --- | --- |
| UART clock | System clock used by the UART timing logic | Defined by the implementation/testbench |
| Baud rate | Serial bit rate | Configurable/selected according to the target system |
| UART frame | Start + data + stop format | Implemented by the UART subsystem |
| AES key size | Master key length | 128 bits |
| AES block size | Data block length | 128 bits |
| CRC width | Checksum width | 16 bits |
| CRC polynomial | Generator polynomial | x^16 + x^12 + x^5 + 1 (0x1021) |
| CRC initial value | Initial register state | 0x0000 |
| CRC input reflection | RefIn | False |
| CRC output reflection | RefOut | False |
| CRC final XOR | XorOut | 0x0000 |

# Literature Review

## 3.1 AES Integration with UART:

Basu, Kole and Rahaman (2012) describe an early implementation of the AES algorithm inside a UART module to secure serial data transfer, showing that block-cipher encryption can be embedded directly in the serial data path rather than performed in software on a host processor. Katkade and Phade (2016) extend this idea, applying AES specifically to protect data confidentiality in serial communication links and reporting on the practical trade-offs of adding a cipher to a low-resource serial interface.

Kala, Panda and Tanwar (2023) target a high-throughput, low-power AES-128 cipher implementation on FPGA, highlighting the area/throughput trade-off between fully unrolled (pipelined) AES round datapaths and iterative, single-round-reuse datapaths. the latter being the approach adopted for the AES-128 core documented in Section 6, where the same physical round data path is reused across all ten rounds under FSM control.

It presents a system that encrypts a 128-bit plaintext block with a fixed AES key for secure UART transfer, reinforcing the practice of pairing AES confidentiality with an explicit, well-documented frame convention the approach followed here with the ciphertext-plus-CRC protected payload.

## 3.2 Error Detection over Serial Links:

Because UART itself provides no automatic-repeat-request or acknowledgment mechanism, corrupted or truncated frames can otherwise be silently accepted by a receiver. Prior work combining UART with CRC-based error detection demonstrates that a cyclic redundancy check substantially improves reliability compared with parity alone, particularly for multi-byte frames where several bits may be corrupted by burst noise. This project uses the CRC-16/XMODEM parameterization specifically polynomial 0x1021 with a non-reflected, zero-initialized register (init 0x0000, RefIn/RefOut false, XorOut 0x0000) — which differs from the closely related CRC-16/CCITT-FALSE variant only in its initial value.

## 3.3 Positioning of This Project:

Relative to the works above, this project's contribution is the coordination of three independently verifiable RTL blocks — an iterative AES-128 core, a byte-oriented CRC-16/XMODEM engine, and a UART transceiver — through an explicit start/busy/done handshake protocol and a top-level FSM, together with a documented verification plan built on standard, published test vectors for both AES-128 and CRC-16/XMODEM.

# System Architecture

The integrated system is organized as a sequence of hardware processing stages. The exact top-level framing can be adapted to the selected application, but the logical relationship between the implemented functions is fixed.

## 4.1 Transmit Path:

Application Data → AES-128 Encryption → CRC-16/XMODEM Generation → UART Transmitter → UART RX

## 4.2 Block-Level Responsibilities:

| Stage | Input | Processing | Output |
| --- | --- | --- | --- |
| Input / Application | Raw application data | Load and validate payload | 128-bit data block / byte stream |
| AES-128 | 128-bit plaintext + 128-bit key | Key expansion + 10 encryption rounds | 128-bit ciphertext |
| CRC-16/XMODEM | Defined protected data field | Polynomial division / CRC update | 16-bit CRC |
| Framing | Ciphertext + CRC | Construct transmission sequence | UART byte stream |
| UART TX | Parallel byte data[7:0] + tx_en request | Frame assembly {stop,data[7:0], start} (LSB-first) and tick-driven bit-serial shift-out via the IDLE/ON FSM | Serial TX bitstream (tx), plus busy/done status |
| UART RX | Serial RX bitstream | Falling-edge detection → 1.5-bit then 1-bit mid-bit sampling via the down-counter → SIPO shift-in → stop-bit validation | Reassembled byte data[7:0], plus busy/done/err status |
| AES-128 Decryption | 128-bit Ciphertext + 128-bit key | Key expansion + 10 Decryption rounds | 128-bit plaintext |
| CRC Check | Data with CRC bits | Polynomial division/check CRC | Valid or corrupted frame |

## 4.3 Positioning of This Project:

A practical protected frame is organized as ciphertext followed by CRC. The exact byte ordering is fixed between transmitter and receiver and documented as part of the interface specification.

The CRC must be calculated over exactly the same byte sequence that the receiver uses for verification. The specification should explicitly define whether the CRC covers plaintext, ciphertext, header fields, or another protected field.

| Field | Size | Purpose |
| --- | --- | --- |
| Ciphertext | 16 bytes | AES-128 encrypted application block |
| CRC | 2 bytes | CRC-16/XMODEM integrity check |
| Total protected payload | 18 bytes | Data transmitted after framing into UART bytes |

## 4.4 Byte Serialization of AES Result:

The 128-bit AES result is converted into 16 bytes for UART transmission. The implementation must define a consistent byte ordering. The most-significant byte is transmitted first, the sequence is Ciphertext [127:120] → Byte 0, Ciphertext [119:112] → Byte 1, Ciphertext [7:0] → Byte 15, followed by the two CRC bytes in the documented order (high byte then low byte).

# UART Communication Module

UART (Universal Asynchronous Receiver/Transmitter) provides the serial communication interface of the system. Because UART is asynchronous at the physical protocol level, the transmitter generates the serial waveform from the local clock while the receiver reconstructs data from the incoming RX signal using its configured bit timing. The system operates on a standard 8-N-1 frame format (1 start bit, 8 data bits LSB first, no parity bit, 1 stop bit) at a default baud rate of 9600 baud using a 50 MHz system clock.

## 5.1 UART Transmitter Operation:

- Idle State: Wait in the idle state (IDLE) with the tx output line held high (logic 1).

- Load Frame: Construct 10 – bit frame {1, data, 0} with start (0), 8 data bits, and stop (1) bits.

- Transmit: Transition to ON state, assert busy, and output bits sequentially indexed by baud rate tick pulses.

- Complete: clear busy, pulse done high, reset index, and return to IDLE after transmitting all 10 bits.

The busy signal is important at the system level: it prevents a new byte from overwriting the currently transmitted byte. The done signal can be used by the integrated controller to advance to the next byte.

## 5.2 UART Receiver Operation:

- Start Detection: monitor rx in IDLE; assert falling edge on a high to low transition.

- Mid-Bit Sampling: Trigger the baud counter to wait 1.5-bit period to sample the stat bit midpoint, then reload 1-bit period for subsequent Data bits.

- Data Shift: Assert en_shift signal on each finish tick to shift incoming rx bits LSB-first into the SIPO register.

- Stop Bit & Validation: Sample the stop bit after 8 data bits; assert done if rx is high or enter ERR state and assert err if low.

## 5.3 UART Interface:

The UART interface is byte-oriented, while AES operates on 128-bit blocks and CRC produces a 16-bit result. The top-level controller is therefore responsible for converting the block-level results into an ordered byte stream.

| Signal | Direction | Description |
| --- | --- | --- |
| clk | Input | System clock signal (50 MHz) |
| rst | Input | synchronous active-high reset signal |
| arst_n | Input | Asynchronous active-low reset signal |
| tx_en | Input | Enable signal to initiate transmission of loaded byte |
| data (TX) | Input | 8-bit parallel byte to transmit |
| tx | Output | Serial UART transmit line |
| busy (TX) | Output | High while a byte transmission is in progress |
| done (TX) | Output | Pulse high indicating completion of byte transmission |
| rx_en | Input | Enable signal for receiver module |
| rx | Input | Serial UART receive line |
| data (RX) | Output | 8-bit recovered received data byte |
| done (RX) | Output | Pulse high indicating valid byte successfully received |
| busy (RX) | Output | High while frame reception is in progress |
| err | Output | indicates a framing error (invalid stop bit) |

## 5.4 UART Testbench:

The provided test verifies the full loopback functionality of the top-level UART module. This is done using a 50 MHz clock setup.

| Test | Input Data | Expected received data |
| --- | --- | --- |
| TEST 1 | 8'hA5 | 8'hA5 |
| TEST 2 | 8'h55 | 8'h55 |
| TEST 3 | 8'hFF | 8'hFF |

# AES-128 Encryption Module

AES (Advanced Encryption Standard) is used to provide confidentiality. The implemented AES engine operates on a 128-bit plaintext block and a 128-bit master key, producing a 128-bit cipher text block. AES-128 uses 10 encryption rounds in addition to the initial AddRoundKey operation.

## 6.1 AES-128 Encryption Structure:

| Operation | Purpose |
| --- | --- |
| Initial AddRoundKey | XOR the plaintext state with the initial round key. |
| Rounds 1–9 | SubBytes → ShiftRows → MixColumns → AddRoundKey. |
| Round 10 | SubBytes → ShiftRows → AddRoundKey; MixColumns is omitted. |

## 6.2 AES State Transformation:

- SubBytes: substitutes each byte using the AES S-box.

- ShiftRows: cyclically shifts rows of the 4×4-byte state.

- MixColumns: applies the AES matrix transformation to each column.

- AddRoundKey: XORs the state with the current 128-bit round key.

## 6.3 AES Key Expansion:

The AES-128 key schedule expands the 128-bit master key into eleven 128-bit round keys, K0 through K10. K0 is used by the initial AddRoundKey operation, while K1 through K10 correspond to encryption rounds 1 through 10.

AES-128 key schedule

Master Key -> K0

|

K1 -> Round 1

K2 -> Round 2

...

K9 -> Round 9

K10 -> Round 10

For encryption, the round-key order is therefore K0 at the initial transformation followed by K1, K2, …, K10. For a corresponding AES decryption implementation (noted as future work in Section 11), the round keys are consumed in reverse order, subject to the exact inverse-round architecture chosen.

## 6.4 AES Control Sequence:

| State | Action |
| --- | --- |
| IDLE | Wait for a new encryption request. |
| LOAD | Capture plaintext and master key. |
| KEY_EXPANSION | Generate/store the AES round keys. |
| INITIAL_ADD_KEY | Perform plaintext XOR K0. |
| ROUND_1 – ROUND_9 | Execute the standard full AES round. |
| ROUND_10 | Execute the final AES round without MixColumns. |
| DONE | Register the ciphertext and signal completion. |

A clocked FSM-based implementation is suitable for this architecture because it makes the sequencing of the ten AES rounds explicit and permits the same physical round datapath to be reused across rounds, minimizing hardware area compared with a fully unrolled implementation

## 6.5 AES Interface

| Signal | Description |
| --- | --- |
| clk | System clock |
| rst_n | Active-low reset, according to the implemented interface |
| start | Starts an AES encryption operation |
| plaintext[127:0] | 128-bit input data block |
| master_key[127:0] | 128-bit AES-128 master key |
| ciphertext[127:0] | 128-bit encrypted output block |
| busy | Indicates that AES is processing |
| done | Indicates that ciphertext is valid |

## 6.6 AES Testbench:

The provided testbench (tb_AES_FSM) verifies the functional correctness and state-machine control flow of the core AES-128 Encryption engine (AES_FSM). It applies clock generation, synchronous reset sequencing, input data latching, and execution pulse triggers while dynamically monitoring simulation signals and reporting final ciphertext results.

Reset Phase: All control inputs (start, plaintext, master_key) are initialized to zero, and active-low reset (rst_n = 0) is held low for two clock cycles before release (rst_n = 1).

Data Latching Phase: 128-bit test vectors for plaintext and master_key are assigned across a clock edge using non-blocking assignments (<=) with a #1 delta delay to prevent setup time violations.

Execution Pulse: The start signal is pulsed high for exactly one clock cycle to transition the FSM from AES_IDLE into AES_ADD_ROUND_KEY_0, initiating the key expansion and 10-round substitution -permutation operations

Completion Synchronization: The testbench halts procedural execution using wait(done); until the FSM completes all 10 rounds and asserts the single-cycle done completion signal.

| Signal Parameter | Bit Width | Expected value |
| --- | --- | --- |
| Plaintext Input | 128 bits | 128'h00000000_00000000_00000000_00000000 |
| Master Key Input | 128 bits | 128'h00000000_00000000_00000000_00000000 |
| Expected Cipher text Output | 128 bits | 128'h66e94bd4_ef8a2c3b_884cfa59_ca342b2e |

# CRC-16/XMODEM Module

The CRC module implements a CRC-16/XMODEM error-detection algorithm in synthesizable Verilog HDL. Its purpose within the complete UART/AES/CRC system is to provide error detection for transmitted data. While AES-128 provides confidentiality, CRC provides a mechanism for detecting accidental corruption of the transmitted information. The implemented module accepts a 128-bit input data word and generates a 16-bit CRC value, resulting in a 144-bit output containing the original data followed by its calculated CRC. The module is parameterized to allow the data width and CRC width to be modified.

## 7.1 CRC-16/XMODEM Parameters:

| Parameter | Value |
| --- | --- |
| CRC width | 16 bits |
| Polynomial | 0x1021 |
| Polynomial expression | x^16 + x^12 + x^5 + 1 |
| Initial value | 0x0000 |
| RefIn | False |
| RefOut | False |
| XorOut | 0x0000 |
| Check value | 0x31C3 for the standard ASCII string “123456789” |

## 7.2 CRC Calculation Concept:

The CRC engine treats the input as a sequence of bits/bytes and updates a 16-bit register using polynomial division. For a non-reflected CRC-16/XMODEM implementation, each input byte is processed from the most significant bit toward the least significant bit.

## 7.3 CRC Interface:

| Signal | Description |
| --- | --- |
| clk | System clock |
| rst_n | Reset input according to the implementation |
| data_in | Input data block |
| busy | High while the CRC calculation is active |
| done | High/pulsed when the CRC result is available |
| data_with_crc | Calculated CRC-16/XMODEM value |

## 7.4 CRC Processing Architecture:

The CRC generator employs a sequential bit-by-bit architecture to calculate the CRC-16/XMODEM checksum over a 128-bit input data block.

The input data is first captured in an internal register, while a 16-bit CRC register stores the intermediate checksum and a counter tracks the number of processed bits. The CRC calculation is initialized to 0x0000 and performed MSB-first, starting from data_in[127] and progressing to data_in[0]. One input bit is processed per clock cycle. For each bit, the feedback value is obtained by XORing the current MSB of the CRC register with the input bit. The CRC register is then shifted left, and the generator polynomial 0x1021 is XORed with the result when the feedback value is logic 1.

The operation is implemented within a single clocked process without a dedicated FSM. When the module is idle (busy = 0), the input is captured and the CRC calculation is initialized. While busy = 1, the module processes one bit per clock cycle until all 128 bits have been evaluated, requiring 128 processing cycles. Upon completion, the calculated 16-bit CRC is appended to the original 128-bit data to form a 144-bit output frame, with the data occupying the upper 128 bits and the CRC occupying the lower 16 bits. The done signal indicates completion and the availability of the valid output, while busy indicates that the CRC engine is actively processing a data block.

## 7.5 CRC Testbench:

The provided testbench verifies the RTL using three independent 128-bit input vectors.

The testbench first applies reset:

After reset is released, each test waits until the CRC module is idle before applying a new input. This is important because the module automatically begins a new calculation when busy is low.

The testbench then waits for the done event:

Once done is asserted, the testbench compares: data_with_crc[15:0 ] against the expected CRC value.

| Test | Input Data | Expected CRC |
| --- | --- | --- |
| TEST 1 | 31323334353637383941424344454630 | 16'h2952 |
| TEST 2 | 00112233445566778899AABBCCDDEEFF | 16'h1248 |
| TEST 3 | 112233445566778899AABBCCDDEEAA55 | 16'h8169 |

# End-to-End Data Path and Integration

The integrated system connects the three implemented processing functions through a top-level control layer. The central integration problem is not the individual algorithms but the movement of data between modules with different granularities: AES processes 128-bit blocks, CRC produces a 16-bit checksum, and UART transmits individual serial frames containing one byte at a time.

## 8.1 Recommended Transmission Sequence:

- Load the application plaintext and AES master key.

- Start AES encryption.

- Wait until AES asserts done and ciphertext is valid.

- Provide the protected data field to the CRC engine.

- Start the CRC calculation.

- Wait until CRC asserts done and crc_out is valid.

- Build the transmission sequence from ciphertext bytes and CRC bytes.

- Transmit each byte through UART only when UART busy is low.

- Wait for UART done before advancing to the next byte.

- After the final byte, return the integrated controller to its idle state.

## 8.2 Integrated State Machine:

| State | Action | Exit Condition |
| --- | --- | --- |
| IDLE | Wait for system start/request | Start asserted |
| AES_START | Launch AES | AES busy |
| AES_WAIT | Wait for ciphertext | AES done |
| CRC_START | Launch CRC | CRC busy |
| CRC_WAIT | Wait for checksum | CRC done |
| UART_LOAD | Select next byte | UART not busy |
| UART_SEND | Request transmission | UART busy |
| UART_WAIT | Wait for byte completion | UART done |
| COMPLETE | Report complete frame | Return to IDLE |

# Control and Handshake Strategy

A clean handshake protocol is essential because each processing block requires multiple clock cycles. The top-level controller should never assume that an output is available merely because an operation was started.

| Signal Pattern | Meaning | Integration Rule |
| --- | --- | --- |
| start | One operation requested | Assert only when the target block is ready/idle |
| busy = 1 | Block is processing | Do not overwrite its active input/result registers |
| done = 1 | Operation completed | Capture/use the result and advance the controller |
| valid | Output data is valid | Consume only when asserted |
| reset | Return to known state | Clear active operations and status flags |

## 9.1 Busy Protection:

The busy signal is especially important for UART transmission. A byte must not be loaded into the UART transmitter while a previous byte is still being serialized. The same principle applies to AES and CRC: their input registers must not be changed during an active operation unless the module explicitly supports pipelining.

## 9.2 Done Pulse Handling:

If done is implemented as a one-clock pulse, the controller should sample it synchronously and transition immediately to the next state. If done remains asserted until acknowledged, the controller must include an acknowledgment or state transition that prevents repeated consumption of the same result.

# AES Integration with CRC

In order to integrate the two blocks together in a wrapper, we need to use a signal which is considered as a handshake signal so when the encryption is already done, cyclic redundancy check is ready for its process.

# Timing and Synthesizability Considerations

The system is clocked RTL and is intended to be synthesizable. Because AES and CRC contain iterative logic and UART contains baud-rate timing, the main timing considerations are combinational depth, register placement, and the number of clock cycles allocated to each operation.

## 10.1 AES Timing:

- An iterative AES architecture reduces hardware area by reusing the round datapath.

- The critical path can be influenced by the S-box implementation, MixColumns logic, and key-expansion logic.

- Registering intermediate results between rounds can improve timing at the cost of additional latency.

- A fully combinational 10-round implementation may reduce cycle latency but typically increases area and timing pressure.

## 10.2 CRC Timing:

- A bit-serial CRC implementation uses very little hardware but requires multiple cycles per byte.

- A byte-parallel implementation can process one byte per cycle but uses more combinational logic.

- The selected implementation is matched to the required throughput of the UART link.

## 10.3 UART Timing:

- UART baud timing must be derived consistently from the system clock.

- The transmitter must maintain the configured bit period with sufficient accuracy.

- The receiver sample near the center of each bit period for robust operation.

## 10.4 Reset Strategy:

Reset behavior initialize all state machines, counters, shift registers, busy flags, done flags, and data-valid indicators. After reset, the integrated system returns to a defined idle state without starting an unintended encryption, CRC calculation, or UART transmission.

# Design Decisions and Engineering Rationale

| Decision | Rationale |
| --- | --- |
| Modular RTL | Allows AES, CRC, and UART to be verified independently and reused in other systems. |
| AES-128 | Provides a standardized 128-bit block cipher with a compact key size suitable for an RTL learning/project implementation. |
| CRC-16/XMODEM | Provides a standardized 16-bit checksum with well-defined parameters and a known validation vector. |
| Handshake-based control | Prevents data overwrite and makes multi-cycle processing safe. |
| Byte-oriented UART interface | Matches the natural granularity of UART while allowing the top level to serialize AES/CRC results. |
| Iterative processing | Offers a practical balance between hardware resource usage and latency for a project-scale RTL implementation. |

## 11.1 Security and Integrity:

AES and CRC serve different purposes and should not be treated as interchangeable. AES provides confidentiality when used with a properly managed secret key, while CRC detects accidental transmission errors. CRC is not a cryptographic authentication mechanism and should not be used to establish authenticity or protect against an intentional attacker modifying the message.

## 11.2 CRC After Encryption:

A communication link can corrupt ciphertext bits just as it can corrupt plaintext bits. Computing CRC over the defined transmitted protected data allows the receiver to detect many accidental bit errors before accepting the data as valid.

# Limitations and Future Improvements

- Add a complete AES decryption path for bidirectional secure communication.

- Add a formal frame format with synchronization, length, sequence number, and explicit CRC coverage.

- Add CRC error reporting at the receiver and a defined retransmission policy.

- Optimize the S-box implementation for the selected FPGA/ASIC technology.

- Add assertions and/or formal verification for key control properties such as busy exclusivity and state transitions.

- Add randomized constrained verification and functional coverage.

- Perform post-synthesis timing analysis and document area, power, and maximum-frequency results.

# Final Specifications

| Parameter | Selected Target |
| --- | --- |
| AES | AES-128, 128-bit key, 128-bit block, 10 rounds |
| AES mode | Direct block encryption (no chaining mode) |
| CRC | CRC-16/XMODEM, polynomial 0x1021, initial value 0x0000, RefIn/RefOut false, XorOut 0x0000 |
| Protected payload | 18 bytes (16-byte ciphertext + 2-byte CRC) |
| UART | Single lane, start/data/stop framing, configurable baud rate |
| Control style | Synchronous start/busy/done handshaking between all stages |
| System status | Individual modules implemented and integrated into a complete RTL system |

# Conclusion

The completed project demonstrates the integration of three important digital hardware functions into a single RTL communication system. UART provides the physical serial data transport, AES-128 provides encryption, and CRC-16/XMODEM provides error detection. The resulting architecture illustrates how independently verified RTL blocks can be coordinated through control states and handshaking to form a practical hardware subsystem.

# References

[1] Basu, D., Kole, D. K., Rahaman, H.: “Implementation of AES algorithm in UART module for secured data transfer.” International Conference on Advances in Computing and Communications (2012), pp. 142–145.

[2] Katkade, P., Phade, G.: “Application of AES algorithm for data security in serial communication.” International Conference on Inventive Computation Technologies (2016), pp. 1–5.

[3] Kala, J., Panda, J., Tanwar, L.: “FPGA implementation of a high throughput Low Power Advanced Encryption Standard (AES-128) Cipher.” 10th International Conference on Signal Processing and Integrated Networks (SPIN) (2023), pp. 367–372.

[4] Kinshuk et al.: “Promoting Sustainable Security by Implementing AES Algorithm for Enhanced Confidentiality in UART Serial Communication.” Springer Nature Link.

[5] National Institute of Standards and Technology (NIST): “Advanced Encryption Standard (AES), FIPS PUB 197.” U.S. Department of Commerce.

[6] CRC-16/XMODEM parameterization: polynomial 0x1021, init 0x0000, RefIn/RefOut false, XorOut 0x0000, check value 0x31C3, as catalogued in standard CRC parameter registries.